Confidential

Stylistic 1200

BIOS 1.11 Installation Instructions

Fujitsu Personal Systems, Inc.

February, 98

Hardware Requirements

To install BIOS Version 1.11 you need the following:

- User BIOS upgrade diskette

- Stylistic 1200 computer with bootable HDD with Windows for Workgroups or Windows 95

- Stylistic external keyboard or compatible PS/2-style keyboard

- Stylistic 1200 port replicator

- Stylistic external floppy disk drive

- AC adapter / power cord

Stylistic 1200 BIOS 1.11 Upgrade Procedure

1.	Attach the port replicator to the Stylistic 1200

2.	Attach the AC adapter / power cord, external keyboard, and external floppy drive to the Stylistic 1200

3.	Boot the Stylistic 1200 and run BIOS Setup by pressing F2 or double tapping on the screen when prompted.

4.	Configure the following BIOS parameters:

	Boot Options: [Boot Sequence is A: then C:]

	Power Savings: [Off]

5.	Save changes and exit BIOS Setup  after the parameters listed are configured. After the changes are saved the system will reboot.

6.	To install BIOS V1.11 you will need to make the Stylistic 1200 BIOS V1.11 diskette into a bootable diskette.   To do this perform the following steps:

	Run MS-DOS and insert the Stylistic 1200 BIOS V1.11 Upgrade Diskette into the external floppy diskette drive.  Make sure that the diskette is not write protected.  NOTE:  If you are using Windows 95, you can open an MS-DOS box to run MS-DOS.

	At the MS-DOS C: prompt type SYS A: and press ENTER.  The system files are copied to the floppy diskette and the message “System transferred” appears.

7.	Leave the diskette in the floppy drive and reboot the system.  The system boots from the floppy drive and the following selection menu is displayed:

- Upgrade Stylistic 1200 to BIOS Version 1.11

- Quit

Choose a selection:

8.	Enter 1 to proceed with the upgrade.

The system displays a window describing the steps taken to upgrade the BIOS.  As the BIOS installation proceeds, a status bar on the screen displays the percentage of ROM erased.  After the BIOS has been erased, another status bar will appear indicating the amount of ROM programmed.

- When the BIOS installation process is complete (usually less than a minute) the system displays the following message.

PhoenixPhlash Status

Flash memory has been successfully programmed

PRESS ANY KEY TO RESTART THE SYSTEM

If the system does not restart

TURN THE POWER OFF, THEN ON

Press the space bar to reboot the system. Note: The FDD  may continue to spin.

- While the system is performing the POST (Power On Self Test), remove the diskette from the floppy drive so that the system boots from drive C.

- Watch the screen during the boot process and observe that the BIOS Version displayed is now Stylistic 1200 Version 1.11.

The BIOS upgrade is complete.