���Ŀ����Ŀ�Ŀ  �Ŀ    ����Ŀ����Ŀ�Ŀ�Ŀ�����Ŀ����Ŀ����Ŀ

�� �;� ջ �� ���� ������ ջ �� ջ �� �� ��ͻ �;� ��;� ջ ��

�� �� � � �� ���� ������ �վ� �� �� �� ���� ���� �Ͱ�� �վ�

�� �Ŀ� ջ �� �Ŀ� �Ŀ���ջ ��� � �� � ���� ���� ��Ŀ�ջ ���

���;�;�;���;���;  Ⱦ�; ����;����;  �;  ����;Ⱦ�;

Communications Utility for DOS

Copyright 1992-1993 RBK Software

All Rights Reserved

* Unregistered Shareware Copy *

For support or the latest version call the

RBK Support BBS at (516) 859-1078

Version 1.6  Users Guide

Contents

License and Copyright Information - page 1

Limited Warranty Information - page 1

System Requirements - page 2

Program Overview - page 2

New Features - page 2

Starting Call Router - page  3

Errorlevel Table - page 6

Call Router Ring Sampler Program - page 7

Creating the Startup Batch File - page 10

RBK Utilities - page 14

DSPDAT16 - page 14

DTSTAMP - page 14

WARMBOOT - page 15

RFDIR - page 15

LAPSE - page 15

DOCURS - page 16

Getting help - page 17

Call Router v1.6 License & Copyright Information

Call Router Copyright 1992-93 RBK Software.

All Rights Reserved.

Call Router is user supported software. If you use this

program beyond a two-week trial period, we ask you to

register your copy.

Benefits of registering Call Router:

1) You help fund the future development of this software.

2) You receive full program support.

3) You receive the next version through U.S. mail with

several utilities to use with Call Router.

Limited Warranty

RBK Software specifically disclaims any and all warranties,

expressed or implied, including fitness for a particular

purpose. Use this product at your own risk.

Distribution

This program may be copied and distributed to others to use

on a trial basis so long as all files in the original archive

are intact and unaltered.

----------

MS-DOS is a trademark of Microsoft Corporation.

Hayes is a trademark of Hayes Microcomputer Products, Inc.

Wildcat! copyright Mustang Software, Inc.

1

Call Router System Requirements

* IBM PC or compatible running MS-DOS 3.1 or higher.

* RS-232 Cable with pin 22 connected. (Ring Indicator)

* Voice grade telephone connection.

* Hayes compatible modem.

Call Router Overview

Call Router is a communications utility which initiates batch

action based on incoming phone rings. Call Router will wait

for a call, set the DOS errorlevel and then exit.

Based on the errorlevel set by Call Router, incoming calls

can be routed to a BBS or perform certain actions according

to how many rings there were, the cadences of the rings, or a

combination of both the ring counts and the cadences detected.

If distinctive ringing service is available on a phone line,

Call Router can detect which number was dialed and set the

errorlevel according to a specific ring cadence.

Distinctive ringing service may also be called Custom Ringing,

Identi-Call, Identi-Ring, MultiRing, Personal Ring, Ring Mate,

Smart Ring, Star Ring or similiar names in different areas.

New features in Call Router v1.6

* Call Router may now be set to answer and establish a data

connection. Once a connection is established, the

errorlevel can be tested to determine the baud rate.

* The Call Router Ring Sample program included in the

distribution archive may be used to create up to three ring

data definition files. Each definition can be used to

represent a different distinctive ring phone number to

allow better customization. This feature now enables Call

Router to recognize three different ring patterns and

process each number uniquely.

* Call Router can now make a full distinctive ring type

determination at the completion of just one ring.

2

Starting Call Router

Calling Syntax...

Call Router should be called from within a batch file (see

section on creating batch file) to monitor incoming calls on

a continual basis and so the errorlevel can be tested upon

exit.

CALLROUT n [speed s] [rings r] [answer] [rm]

[userdata] [ssoff] [dtron] [pf]

Arguments enlosed in [] are optional and may be in any order.

You may type: CALLROUT<Enter>, without arguments, to display

the program usage from the DOS prompt.

Description of arguments...

The first argument must always be the number of the COM port

Call Router should open.

n - where argument n is COM port 1,2,3, or 4.

This argument must always be the first.

speed s - Use this argument when the COM port needs to be

opened at a specific baud rate. Call Router will

default to 2400 baud when argument is not used.

rings r - Determines number of rings Call Router should count

before setting errorlevel and exiting. Call Router

will default to 2 rings when argument is not used.

Ignored with 'userdata' argument.

answer - Send answer command to modem.

Call Router will wait for carrier detect. When a

connection has been established, the program will

send a title screen to the caller including a

"Loading program..." message.

Call Router then exits and sets the errorlevel

based on the baud rate detected. From this point

the batch file should be set up to call a host or

BBS program. Ignored with 'userdata' argument.

3

Starting Call Router (cont.)

rm - Sets ringmate detection on.

In previous versions, Call Router had been primarily

designed to work with the distinct ring service

offered by New York Telephone known as Ring Mate.

Their distinct rings are shorter in duration than

standard rings. When this argument is found, Call

Router will exit and set errorlevel to 4 if Ring Mate

type rings are detected and 3 if standard rings are

detected. This argument has been retained to keep

version 1.6 compatible with older versions.

Ignored with 'userdata' argument.

userdata - Use of this argument will instruct Call Router to

search for data files created by the CRSAMPLE

program. The CRSAMPLE program allows you to create

up to 3 ring cadence definitions. Each data file

contains the following information:

1) Phone number

2) Should call router answer?

3) Exit on which ring?

4) Ring data

5) Rest data

As Call Router detects a call, it will procede as

specified in the data file for the specific call

pattern detected.

* The rings, answer, and rm, arguments are ignored

when the userdata argument is used.

dtron - Set DTR to ON upon exit.

Unless Call Router has answered and is exiting so

another program can take control of the call, DTR is

set OFF or LOW. When Call Router is not set to

answer, use this argument to leave DTR on when Call

Router exits.

4

Starting Call Router (cont.)

ssoff - Disables screen-saver.

To prevent screen burn-in, Call Router will

automatically blank the screen while waiting for a

call if there is no call or keyboard activity for

sixty seconds. Use this argument only if you wish to

disable this feature.

pf - Print form.

When this argument is detected, a prompt asking if

a registration form should be printed appears.

After the form is printed or the request to print

is aborted, Call Router continues as normal.

5

Errorlevel Table

Refer to this table when setting up your batch file to test

the errorlevel condition Call Router sets upon exit. Please

notice that the possible errorlevel condition is largely

based on the arguments used start to Call Router.

The symbols 'w/RM' and 'w/USERDATA' indicate the possible

outcome when used with the RM or with the USERDATA arguments.

Errorlevel   Condition

----------   ---------

0       Normal termination. ESC key pressed.

1       Fatal Error. Incorrect usage or errors upon

initialization.

2       Timeout. Time period between rings or wait for

carrier took too long.

3       Standard ring detected.

w/RM: Standard ring detected

w/USERDATA: match definition file 1, no answer.

4       w/RM: Ring Mate ring detected.

w/USERDATA: match definition file 2, no answer.

5       w/USERDATA: match definition file 3, no answer.

10       w/USERDATA: match definition file 1, 300 BPS.

11       w/USERDATA: match definition file 1, 1200 BPS.

12       w/USERDATA: match definition file 1, 2400 BPS.

13       w/USERDATA: match definition file 1, 4800 BPS.

14       w/USERDATA: match definition file 1, 9600 BPS.

15       w/USERDATA: match definition file 1, 12000 BPS.

16       w/USERDATA: match definition file 1, 14400 BPS.

20       w/USERDATA: match definition file 2, 300 BPS.

21       w/USERDATA: match definition file 2, 1200 BPS.

22       w/USERDATA: match definition file 2, 2400 BPS.

23       w/USERDATA: match definition file 2, 4800 BPS.

24       w/USERDATA: match definition file 2, 9600 BPS.

25       w/USERDATA: match definition file 2, 12000 BPS.

26       w/USERDATA: match definition file 2, 14400 BPS.

30       w/USERDATA: match definition file 3, 300 BPS.

31       w/USERDATA: match definition file 3, 1200 BPS.

32       w/USERDATA: match definition file 3, 2400 BPS.

33       w/USERDATA: match definition file 3, 4800 BPS.

34       w/USERDATA: match definition file 3, 9600 BPS.

35       w/USERDATA: match definition file 3, 12000 BPS.

36       w/USERDATA: match definition file 3, 14400 BPS.

6

Call Router Ring Sample Program version 1.0 beta

Version 1.6 has the ability to search for pre-defined ring

definition data files when the 'userdata' argument is used.

To create these data files, a supplementary program named

CRSAMPLE.EXE has been included in this archive. Up to three

data files may be created, each representing a different

phone number a caller may dial. A minimum of two must be

found to use the 'userdata' option.

This new feature allows you to specify whether or not Call

Router should answer a call or not and how many rings it

should wait before answering that particular number.

To define the ring pattern for each phone number you wish to

use, a sample ring and rest period are measured. This

requires that each number be dialed remotely.

Using the CRSAMPLE program...

From the DOS prompt type: "CRSAMPLE n" followed by enter.

where n is the COM port you wish to use -> 1, 2, 3, or 4.

If you have provided a valid COM port for the program to use,

it will continue by presenting the introductory screen. After

you have read the information presented, press a key to

continue.

At this point, the CRSAMPLE program will perform a search for

three files: CR16_1.DAT, CR16_2.DAT, and CR16_3.DAT. The

program will display the search results and provide four

options for you to take. By choosing options 1, 2, or 3 you

may create or modify the data files. Choosing option 4 quits

the program. This is the main screen where the program

returns after a ring definition is created and sampled.

To create a new definition file or to replace an existing

file, choose 1-3 based on which file you would to create or

modify. Use 1 for CR16_1.DAT, 2 for CR16_2.DAT, and 3 for

CR16_3.DAT. For Call Router to operate correctly with the

userdata option, it is important to remember that there may

be only ONE definition file per phone number. It does not

matter which file you assign a phone number to.

7

Call Router Ring Sample Program version 1.0 beta (cont.)

The ring definition files must be created in order. To create

CR16_3.DAT, CR16_1.DAT and CR16_2.DAT must be created first.

Likewise, CR16_1.DAT must be created prior to creating

CR16_2.DAT.

This is a fairly simple procedure and is explained in detail below.

"Continuing will delete previous data in file , O.K. (Y/N)?"

When you choose to modify an existing data file you will be

warned that all previous data written to this file will be

deleted. Enter a 'Y' if you wish to continue or a 'N' to

abort. If no data file had been found, this question is

skipped.

You will be asked to enter the phone number of the rings to

be be sampled.

"Enter phone number for file # -> "

This may be any group of characters up to 24 you wish to use

to identify a particular number.

"Do you want Call Router to answer and establish a data

connection when 555-5555 is detected? (y/n) "

Answer 'y' if you want Call Router to answer, 'n' if not.

"Wait/Answer on ring number (1-99)? "

Enter the number of rings you wish Call Router to count

before processing a call detected on this phone number.

"Press a key to start sample procedure..."

You are now ready to capture a ring sample for this number.

If you have not yet pressed a key to advance do so now, you

should have the message:

"Waiting for call..."

8

Call Router Ring Sample Program version 1.0 beta

At this point the number being defined may be dialed from a

remote location and allowed to ring at least two times. You

may also press ESC to abort at this time if you wish. When

the ring data is measured you will see,

"Ring data sucessfully captured"

or if the procedure was incomplete,

"The sampling procedure was incomplete"

Pressing a key will return you to the main menu where the

screen will be updated with the new file information. If

there was an existing file you attempted to modify and the

ring sample procedure was incomplete, you will see that the

file was now gone. Otherwise, you will see a newly created

file or the same screen if the file had existed before.

Repeat the procedure for up to three phone numbers. If you

are sampling two numbers, you should only have two

data files. To delete a ring definition you may enter the

proper number to modify and simply hit ESC at the sample

procedure which will make the procedure incomplete and delete

the file. You may also use the ERASE or DELETE command at the

DOS prompt to delete a ring definition.

Registered Users Note...

If you wish to view the contents of the Call Router v1.6 data

files, there is also a display program DSPDAT16.EXE included

with this archive. By typing "DSPDAT16" followed by enter at

the DOS prompt, you may view the contents of the data files

created by the CRSAMple program.

9

Creating the Startup Batch File

As in earlier versions, you need to create a startup batch

file. This batch file starts Call Router and tests the

errorlevel upon exit. It also calls the proper application

program and can be set up to allow Call Router to monitor

calls on a continual basis.

There are two batch files (.BAT extension) included with

Call Router. They are example files to show how Call Router

may be used. Please feel free to modify these files to suit

your own particular application.

As an example, we can look at MULTI.BAT (included with Call

Router distribution files) line by line. This setup shows how

a single phone line with distinctive ring service, with two

additional numbers added, may be used to run a two BBS systems.

In this example, assume the following numbers are used:

555-1111  Main number we use for voice.

555-2222  Distinctive ring phone #1.

This is the number we give out for the Wildcat BBS.

555-3333  Distinctive ring phone #2.

This is the number to be called for some other

application.

Run CRSAMPLE to obtain data for each number...

1. Sampled main number to be used for voice.

Answer = 'N'

Set ring count to 99 so Call Router continues to

count rings until someone answers or callers hangs up.

2. Sampled first additional distinctive ring for WILDCAT BBS.

Answer = 'Y'

Set ring count to 1.

3. Sampled second additional distinctive ring for MY APP.

Answer = 'Y'

Set ring count to 1.

4. Create BBS.BAT...

10

Creating the Startup Batch File (cont.)

The batch file "MULTI.BAT" included with the Call Router

archive is similiar to batch file shown below and may be

copied and edited.

Please note that the text to the right of the '<' are

comments and would normally be omitted.

--------------------------------------------------------

@echo off  <-- This line turns off echoing commands to display

:START     <----------- START Label

CALLROUT 1 USERDATA  <- Start Call Router using COM 1, use

<  data files created by CRSAMPLE program.

if errorlevel 36 goto MYAPP14400  < These lines

if errorlevel 35 goto MYAPP12000  < test to see if

if errorlevel 34 goto MYAPP9600    < phone #3 was answered.

if errorlevel 33 goto MYAPP4800    < If so,

if errorlevel 32 goto MYAPP2400    < the appropriate

if errorlevel 31 goto MYAPP1200    < command is called to

if errorlevel 30 goto MYAPP300      < start 'MY APPLICATION'.

if errorlevel 26 goto WC14400       < These lines

if errorlevel 25 goto WC12000       < test to see if

if errorlevel 24 goto WC9600         < phone #2 was answered.

if errorlevel 23 goto WC4800         < If so,

if errorlevel 22 goto WC2400         < the appropriate

if errorlevel 21 goto WC1200         < command is called to

if errorlevel 20 goto WC300           < start the WILDCAT BBS.

< * Note: testing for errorlevels 5 and 4 would be if inserted

<            here if answer was set to NO in definitions 2 and/or 3.

if errorlevel 3 goto START <-- Standard call reached 99 rings.

<   Highly unlikely, buts prevents

<   false indications if batch

<   loops around before caller

<   hangs up.

< continued...>

11

Creating the Startup Batch File (cont.)

if errorlevel 2 goto START <-- Timeout condition. Occurs when

<   caller hangs up before ring

<   count was reached or able to

<   detect carrier during answer.

if errorlevel 1 goto FATAL <-- A fatal error!

if errorlevel 0 goto END   <-- The ESC key was pressed.

:MYAPP14400                < MYAPP is called with the

CALL MYAPP /B 14400   < appropriate baud rate.

if errorlevel 1 goto END   < Assume MYAPP exits at

goto START                  < errorlevel 1 on error.

:MYAPP12000

CALL MYAPP /B 12200

if errorlevel 1 goto END

goto START

:MYAPP9600

CALL MYAPP /B 9600

if errorlevel 10 goto END

goto START

:MYAPP4800

CALL MYAPP /B 4800

if errorlevel 10 goto END

goto START

:MYAPP2400

CALL MYAPP /B 2400

if errorlevel 10 goto END

goto START

:MYAPP1200

CALL MYAPP /B 1200

if errorlevel 10 goto END

goto START

:MYAPP300

CALL MYAPP /B 300

if errorlevel 10 goto END

goto START

< continued... >

12

Creating the Startup Batch File (cont.)

:BBS14400                       <  Wildcat is called with the

CALL WILDCAT /B 14400   <  appropriate baud rate.

if errorlevel 60 goto START <  Wildcat exits at

goto END                        <  errorlevel 60.

:BBS12000

CALL WILDCAT /B 12200

if errorlevel 60 goto START

goto END

:BBS9600

CALL WILDCAT /B 9600

if errorlevel 60 goto START

goto END

:BBS4800

CALL WILDCAT /B 4800

if errorlevel 60 goto START

goto END

:BBS2400

CALL WILDCAT /B 2400

if errorlevel 60 goto START

goto END

:BBS1200

CALL WILDCAT /B 1200

if errorlevel 60 goto START

goto END

:BBS300

CALL WILDCAT /B 300

if errorlevel 60 goto START

goto END

:FATAL    <--- Display message and end on fatal error.

@echo.

@echo Call Router ended with a fatal error.

@echo.

goto END

:END      <--- Nothing to do, end of batch.

---------------------------------------------------------

13

RBK Utilities

If you are a registered user, you received the Call Router

archive with several utilities. Below is a description of

these utilities and how they may used with Call Router.

Data File Displayer for Call Router v1.0 beta

Usage: DSPDAT16

This Call Router specific utility displays the contents of

data files created by the CRSAMPLE program.

Date/Time Stamp v1.1

Usage: DTSTAMP

Date/Time Stamp provides a one-line reading of the date and

time. Unlike the DOS date and time commands, no prompts

follow, allowing uninterrupted use from within a batch file.

DTSTAMP may be used for continuous log entries from within a

batch file.

i.e. The following line shows an example of how DTSTAMP may

be inserted into an autoexec.bat file to keep a running log

of reboots on your system.

dtstamp >> boots.log

This would append "Date 10/1/1993  Time 11:24:00" to the end

of the file boots.log. This may be used similiarly in the

batch file used to start Call Router to record when an

incoming call is detected.

14

RBK Utilities (cont.)

Warm Boot v1.1

Usage: WARMBOOT

Performs a DOS reload without the preliminary hardware reset

and testing. Basically the same as pressing the

<Ctrl+Alt+Delete> keys. This may be placed within a batch

file to reboot when a certain condition is met. For example,

after 10 rings are detected.

Run File Directory v1.1

Usage: RFDIR [path]

where path is any valid DOS filepath.

Use RFDIR to list only executable files in a specific

directory. Only files with the extensions: .COM, .EXE, and

.BAT are included in the listing. This program is

particularly useful when there are a large number of files in

a directory and the executable files need to be sorted out

quickly.

Lapse v1.2

Usage: Lapse [seconds]

where seconds is the number of seconds you wish to

lapse.

Use LAPSE when you want to suspend activity batch file

activity for a given amount of time in seconds. Typing LAPSE

without specifying seconds will default to five seconds. If

LAPSE is allowed to countdown it will exit errorlevel 0. If

<Ctrl+Break> or <Ctrl+C> is detected, LAPSE will exit setting

the errorlevel to 1.

15

RBK Utilities (cont.)

Do Cursor v1.0

Usage: DOCURS

This simple utility resets your cursor size and position to

normal when a program 'forgets' to restore it. You will not

need to use this with Call Router, but it comes in handy when

you need your cursor and don't want to reboot when a program

fails to 'cleanup'.

16

Support

Any questions, suggestions, or comments may be addressed

to any or all of the following locations:

RBK support BBS - 24hrs.

(516) 859-1078 (300/1200/2400-8/N/1)

Compuserve [70714,3020]

U.S. Mail:

RBK Software

Robert Mulligan

280-13 Belmore Avenue

East Islip, NY  11730

17