FIXUMBS 1.00

by

Roland Skinner

Copyright (c) 1993

RJS Software

FIXUMBS is a utility to aid in UMB memory management.  It  is  particularly

useful  when  using  DOS  as  a   UMB-manager,   along   with   third-party

memory-managers which may act as UMB providers.

FIXUMBS was developed to get the  most  out  of  a  system's  memory.  Many

computers do not  have  access  to  the  full  amount  of  physical  memory

installed on those machines. This is mainly due to their BIOSes  not  fully

supporting the motherboards' chipsets or  because  those  chipsets  do  not

support full relocation (mapping) of memory.

Often, many computers with more than 2MB of memory appear to have 384K less

memory than they actually have. This is particularly common with  computers

that have an AMI BIOS.  This  memory  is  usually  set  aside  for  use  as

shadow ram for the systems' ROMs.  (Shadow  ram  is  RAM  that  contains  a

"mirror-image" of a ROM chip's contents that is located in the position  of

that ROM, which improves  system  speed  due  to  RAM  being  an  order  of

magnitude faster than ROM.) Often, the amount of ROM in a computer  is  far

less than 384K, and the remaining RAM set aside for shadowing does not  get

used.

Many utilities have been written to map the remaining shadow ram  into  the

UMB (or upper) memory regions. FIXUMBS is written to aid these utilities to

manage those UMBs. FIXUMBS is probably more useful  when  one  is  using  a

memory manager such as QEMM or 386MAX to provide certain regions  of  UMBs,

especially with products such as QEMM v6.00 (or above) that contain stealth

technology to map more UMB regions into the regions taken up by  the  ROMs.

Now, these memory managers do not recognize most  chipsets,  and  therefore

do not use the shadow ram of the computers that contain those chipsets.

By using these memory managers, along with a shadow ram mapping utility, in

conjunction with FIXUMBS, one can achieve an optimal memory  usage  setting

on one's computer system. On the author's  system,  128K  of  XMS  and  EMS

memory was gained, along with the benefit of having 640000  bytes  of  free

conventional memory, through the use of FIXUMBS and other utilities.

REQUIREMENTS

~~~~~~~~~~~~

- DOS 5 (or above)

- XMS memory manager

- DOS UMB management

- Shadow ram UMB provider recommended

HOW TO INSTALL FIXUMBS

~~~~~~~~~~~~~~~~~~~~~~

The heart of  FIXUMBS  is  the  file  FIXUMBS.EXE.  This  file  is  both  a

device-driver (installed in CONFIG.SYS) and an executable program  (run  at

the command-prompt or a batch file). FIXUMBS should be run twice.  Firstly,

FIXUMBS must be installed as a device-driver in one's CONFIG.SYS file,  and

then be run in the AUTOEXEC.BAT file.

Installation in CONFIG.SYS

~~~~~~~~~~~~~~~~~~~~~~~~~~

FIXUMBS must be installed near the  beginning  of  one's  CONFIG.SYS  file.

It must be installed after the shadow ram UMB-provider and  after  the  XMS

memory manager that provides the other UMBs. Note that DOS must be  set  as

the UMB-manager.

If FIXUMBS determines that one or more of its  requirements  are  not  met,

i.e. DOS is not managing UMBs, an XMS memory manager is not  installed,  or

it cannot find any UMBs (or a big  enough  free  UMB),  then  it  will  not

install and will display an error as to the reason for no installation.

Note that the shadow ram UMB regions must be excluded if one  is  using  an

XMS memory manager to provide other UMBs. Note also that DOS 5  (or  above)

is required, as it must manage the UMB-memory.

The following is an example of a CONFIG.SYS file, using UMB_DRVR to provide

the shadow ram UMBs, QEMM v6+ to provide stealth technology  UMBs  and  DOS

managing the UMBs:

:

:

DOS=HIGH,UMB

:

:

DEVICE=UMB_DRVR.SYS /C=xx /M=--------..--++++++++....

DEVICE=QEMM386.SYS R:2 NOSH ARAM:D000-EFFF RAM ST:M

DEVICE=FIXUMBS.EXE

:

:

Note that FIXUMBS must be loaded low; it will not install if loaded high.

The above example uses to UMB_DRVR to  provide  128K  of  shadow  ram  UMBs

between D0000-EFFFF, and  QEMM  to  provide  stealth  memory  for  the  EMS

page-frame at C0000-CFFFF and a further (approx) 64K of stealth memory  for

more UMBs between F0000-FFFFF. Note that the  ARAM:D000-EFFF  option  above

excludes the shadow ram UMB-memory, as discussed above.

Installation in AUTOEXEC.BAT

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Installation of FIXUMBS in the AUTOEXEC.BAT file is  optional,  but  highly

recommended. At the beginning of the AUTOEXEC.BAT file, one should  include

a line that runs FIXUMBS.EXE. This  installation  does   not  take  up  any

memory, however it provides extra security for FIXUMBS's operation.

FIXUMBS must have been installed in the CONFIG.SYS file prior to  execution

from the AUTOEXEC.BAT (see previous section).

SUPPORT PROGRAMS - FUMBON.COM and FUMBOFF.COM

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

FUMBON.COM and FUMBOFF.COM are utilities to turn  FIXUMBS's  UMB-management

on or off, respectively.

Sometimes, incompatibilities between FIXUMBS and some  other  software  may

arise. These programs  may  be  executed  to  disable  FIXUMBS  operability

temporarily, and then reenable it soon thereafter.

Note that these programs require FIXUMBS to be  installed  prior  to  their

use.

FIXUMBS and MICROSOFT WINDOWS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

FIXUMBS has been  tested  thoroughly  with  Microsoft  Windows  running  in

Enhanced Mode, and has been found  to  be  fully  compatible.  FIXUMBS  has

built-in code to detect  the  initialization  of  Windows  and  temporarily

disables itself during the execution of Windows, and then reenables  itself

after exiting Windows. FIXUMBS is disabled since Windows  needs  to  manage

all memory itself during its execution.

However, Windows running in Standard Mode has had some problems.  Executing

Windows in standard mode  will  cause  one's  system  to  hang  if  special

precautions are not made prior  to  its  use.  Simply  execute  FUMBOFF.COM

before running standard-mode Windows, and all will be  fine.  FIXUMBS  will

reenable itself automatically after Windows is exited.

The incompatibility with standard mode could be corrected if FIXUMBS  could

detect  the  initialization  of   standard-mode   Windows,   as   it   does

enhanced-mode. If anyone knows of such a method, the author  would  greatly

appreciate it.

BENEFITS OF USING FIXUMBS

~~~~~~~~~~~~~~~~~~~~~~~~~

FIXUMBS's major benefit comes from its ability to  emulate  UMB  allocation

and deallocation of an XMS-server, when DOS 5+ is the primary  UMB-manager.

Often, when DOS is managing UMBs, many programs do not use DOS's method  of

allocating UMBs, and thus don't load themselves high. Furthermore,  if  one

is using an XMS manager such as  QEMM,  and  DOS  is  managing  UMBs,  many

programs detect the XMS manager and try to use its UMB allocation  methods,

yet fail because they should have used DOS's methods.

FIXUMBS has been found particularly useful  with  4DOS  (JP  Software)  and

DOSMAX (Philip B. Gardner), in setups that have DOS  UMB  management  along

with third-party UMB providers/managers (where DOS  does  the  management),

where these programs fail to allocate UMBs. These and  other  programs  use

documented methods of allocating UMB memory, yet fail since these  sort  of

setups cause the XMS-server to report  no  available  high-memory.  FIXUMBS

reports the free high-memory as free (when it is free).  Furthermore,  some

programs detect the presence of  third-party  memory-managers,  and  assume

that these memory-managers are managing the UMBs, when in fact they aren't.

Use of FIXUMBS will aid in better UMB management, since it tries to  ensure

that UMBs are their maximum sizes (one large free UMB instead of many small

UMBs), to provide more availability of memory for other programs.

ALREADY TESTED...

~~~~~~~~~~~~~~~~~

FIXUMBS has been thoroughly tested with DOS 6.0, QEMM  v6.02,  4DOS  v4.02,

WINDOWS v3.1, and with UMB_DRVR v5.22. It is assumed that it will work with

DOS 5.0 and  previous  versions  of  the  above  software.  Numerous  other

software has been found  to  work  fine  with  FIXUMBS,  including  certain

protected mode software.

The only known problem (so far) occurs with  running  Windows  in  standard

mode. See above for discussion of correcting  this  problem.  In  a  future

version, it is hoped that FIXUMBS will  be  able  to  detect  standard-mode

Windows automatically, and thus    be  able  to  circumvent  any  potential

problems before they occur.

If anybody finds any problems  with  FIXUMBS  and  other  software,  please

contact the author or RJS Software (see below).

LICENSE and DISCLAIMER

~~~~~~~~~~~~~~~~~~~~~~

FIXUMBS is public domain software. It has been found to achieve the task it

has been written to do. However,  due  to  the  technical  nature  of  this

program, it is near impossible for  all  system  setups  to  be  completely

error-free. The author does not accept any responsibility for any  loss  or

damage that arises out of the use of  this  software,  due  to  correct  or

incorrect use.

The programs and distributed here must be kept together, and should not  be

distributed without this documentation. These files are:

FIXUMBS.EXE, FUMBON.COM, FUMBOFF.COM, FIXUMBS.DOC, FILE_ID.DIZ

If you find FIXUMBS useful, a donation of $5 (US)  or  R10  (SA)  would  be

appreciated. All donations, correspondence, wish-lists, etc. can be sent to

the address(es) below.

The author reserves the right to discontinue support or development of this

software, at any time.

I hope that users of FIXUMBS will find it useful.

Roland Skinner

---------------------------------------------------------------------------

COPYRIGHT INFORMATION

~~~~~~~~~~~~~~~~~~~~~

The author and RJS Software acknowledge  all  copyrights  of  the  software

mentioned here, and also of other software that performs the same tasks  as

the software mentioned here.

---------------------------------------------------------------------------

History of FIXUMBS

-  v1.00 Public release.

First official public release.

---------------------------------------------------------------------------

Any correspondence may be forwarded to the address below.

Send  your  recommendations,  bug-reports,  requests,  donations,  etc.  to

Roland Skinner, or RJS Software, at:

P.O. Box 14134

Bredell

1623

Republic of South Africa

The above address is not a residential address; only a box number.

Users requesting faster replies, etc. may send an Email message  to  Roland

Skinner on one of the echomail conferences in South Africa, or (preferably)

netmail me. My personal Email addresses are:

FIDONET:    Roland Skinner @ 5:7101/55.1

InterNet:   roland.skinner@p1.f55.n7101.z5.fidonet.org

roland@146.141.15.20

roland@146.141.27.64

Roland Skinner