Espaol (see the end of the file for english explanation).

----------------------------------------------------------

2MX permite trabajar en PC/XT equipados con controladoras de alta densidad,

tales como Amstrad 2086 u Olivetti Cuaderno. El nico requisito es equiparlos

tambin con una unidad de alta densidad. En las dems mquinas PC/XT se puede

sustituir la controladora de disquetes por otra de AT (incluso de 16 bits; no

importa que quede la mitad de la misma sin insertar en el slot).   Si la BIOS

no soporta alta densidad, caso ms probable, habr que cargar 2M-XBIOS antes,

as como disponer de disco duro o una unidad A: de doble densidad donde poder

arrancar al principio. Con 2MX se podr adems acceder a los disquetes 2M sin

limitaciones.  Si  la  mquina  posee  un conmutador de turbo,  al cambiar la

velocidad de la CPU se afecta al normal funcionamiento de 2M-XBIOS y 2MX.  Si

hubiera problemas en el acceso a disco, se pueden ejecutar estos programas en

cualquier  momento  para  actualizar las  constantes internas de retardo a la

velocidad actual del sistema.

Con 2M-XBIOS instalado, se pueden formatear disquetes estndar del DOS pero

generalmente indicando el nmero de pistas (/T:40  /T:80) as como el nmero

de sectores  (/N:9, /N:15  /N:18, para discos de 360/720K, 1.2M  1.44M)  al

invocar el programa FORMAT del sistema a partir de la versin 3.30

English

-------

2MX improves 2M diskettes support for your PC/XT, if it has an high density

diskette controller, as Amstrad 2086 or Olivetti Cuaderno.  All what you need

is to buy a high density drive if yours is double density.  The rest of PC/XT

machines may need an AT high density diskette controller (there is no problem

with 16-bit one: insert only half card on one expansion slot)  if you doesn't

find easily a PC/XT card. If BIOS don't support high density (the most common

situation) you must load 2M-XBIOS before and the machine must be equiped with

a  hard  disk  or a double density drive A: where boot from at start up time.

With 2MX loaded on memory, 2M diskettes can be used without any restrictions.

If the computer has a turbo switch, changing the CPU speed may cause abnormal

function  on  2M-XBIOS  and 2MX.  If you detect  any  problem in disk access,

this  programs  can  be  executed  in any moment to update the internal delay

constants to the actual system speed.

With 2M-XBIOS loaded, DOS standard diskettes can be formatted,  but usually

setting the tracks number (/T:40 or /T:80)  and the sectors per track  (using

the /N:9, /N:15 or /N:18 switches in 360/720K, 1.2M or 1.44M diskettes)  when

calling DOS FORMAT program since 3.30 version of DOS.