@(#)hdsinfo.doc  1.4

HDS-300 Commands for IBM PC/AT

==============================

By John Elliott, Motorola Development Systems, Austin, TX

1. Purpose

The HDS-300 Commands for IBM PC/AT provide a useful method for

transferring binary and ASCII files between an IBM PC/AT and a

Motorola HDS-300 Hardware Development Station (CP/M-68K).  This

provides a path for modem file transfers using the PC/AT with

any type of communications software and also allows usage of

more powerful editors for editing HDS-300 macro files.

2. Installation

a. The files needed are listed below.  Delete all prior versions.

Filename      Vers.  Description

------------  -----  -------------------------------------------

HDSINFO.DOC          This information file you're reading now.

HDSDEL.EXE     1.0   Deletes HDS-300 file command.

HDSDIR.EXE     1.0   Directory for HDS-300 diskettes command.

HDSREAD.EXE    1.0   Copies (reads) HDS-300 file to DOS file

command.

HDSTYPE.EXE    1.0   Display HDS-300 file contents command.

HDSWRITE.EXE   1.0   Copies (writes) DOS file to HDS-300

diskette command.

b. An IBM PC/AT (or 100% compatible) with high capacity (1.2 MB)

5-1/4" floppy disk drive is required.

c. Place all the HDS*.EXE files in your execution directory, e.g.

\BIN.  The HDSINFO.DOC file is not required to execute the

commands.  It is for informational purposes only.

-- NOTE --

The MS-DOS "PATH" command must be properly set  so  it

can find the directory where the HDS*.EXE files reside.

3. Bugs Fixed

a. Directory read/write problems have been resolved.

4. Known Bugs

a. HDSWRITE.EXE:

- Does not work reliably if you fill the HDS-300 disk beyond its

storage capacity (624K).

- Does not properly handle subdirectories in the source filename.

- Sometimes gives "File not found" error, when in fact, the file

does exist.  Re-issuing the command always seems to work.

b. HDSDIR.EXE:

- Does not report the proper amount of free space available.  It

shows 4K more than what is actually available.  Remember, disk

space is allocated in 2K "blocks", i.e. a 10 byte file actually

occupies 2K of disk space, a 2050 byte file occupies 4K of disk

space, etc.

5. Command Syntax

The following descriptions are presented in the structure of the

HDS-300 Commands Reference Manual.  If any command is invoked

without any arguments, a one-line command syntax description along

with the command version number is displayed.  The first release of

these commands did not display any version number when invoked

without any arguments.  Thus version 1.0 is really the second release.

Wildcard characters are permitted in DOS and HDS-300 filenames

as per their usual conventions.

HDSDEL       Delete HDS-300 file

--------------------------------

Description:

The HDSDEL command allows files on a HDS-300 diskette to be

deleted from the diskette.

Syntax:

HDSDEL  <drive>[user]:[<filespec>]

where:

<drive>      selects the IBM PC/AT drive in which the HDS-300

diskette has been inserted.

<user>       is the optional CP/M-68K user number for <filespec>.

Legal values are 0-15, with 0 as the default.

<filespec>   is the HDS-300 filename to be deleted.

Examples:

HDSDEL  A:TEST.MAC   Deletes the TEST.MAC file on the HDS-300

diskette in drive A: of the IBM PC/AT.

HDSDEL  A9:TEST.SCR  Deletes the TEST.SCR file under user 9 on

the HDS-300 diskette in drive A: of the

IBM PC/AT.

HDSDIR       Disk directory

---------------------------

Description:

The HDSDIR command allows filenames on a HDS-300 diskette to

be displayed.

Syntax:

HDSDIR  <drive>[user]:[<filespec>]  [/W]  [/P]

where:

<drive>      selects the IBM PC/AT drive in which the HDS-300

diskette has been inserted.

<user>       is the optional CP/M-68K user number for <filespec>.

Legal values are 0-15.  If omitted, all user numbers

are searched.

<filespec>   is an HDS-300 filename.

/W           is the normal MS-DOS option to produce a wide dis-

play of the directory.

/P           is the normal MS-DOS option to pause when the screen

is full.

Examples:

HDSDIR  A:           Lists all files for all user numbers on the

HDS-300 diskette in drive A: of the IBM PC/AT.

HDSDIR  A3:*.MAC     Lists all macro files under user number 3

on the HDS-300 diskette in drive A: of the

IBM PC/AT.

HDSDIR  B: /P        Lists all files for all user numbers on the

HDS-300 diskette in drive B: of the IBM PC/AT

and pauses when the screen becomes full.

HDSREAD      Copy HDS-300 file to DOS File

------------------------------------------

Description:

The HDSREAD command allows files on a HDS-300 diskette to be

copied to MS-DOS.  It follows a command format similar to the

MS-DOS "COPY" command.

Syntax:

HDSREAD  <drive>[user]:[<filespec>]  <DOS_path>

where:

<drive>      selects the IBM PC/AT drive in which the HDS-300

diskette has been inserted.

<user>       is the optional CP/M-68K user number for <filespec>.

Legal values are 0-15, with 0 as the default.

<filespec>   is an HDS-300 filename to be read.

<DOS_path>   is the MS-DOS destination path to be written.

Examples:

HDSREAD  A:USERINIT.MAC  C:    Copies the USERINIT.MAC file on

the HDS-300 diskette in drive A:

of the IBM PC/AT to a DOS file

on drive C:.

HDSREAD  A6:TMP.MAC  TST.MAC   Copies the TMP.MAC file under user

number 6 on the HDS-300 diskette

in drive A: of the IBM PC/AT to a

DOS file named TST.MAC on the

default drive.

HDSREAD  B:BIG.MAC  .          Copies the BIG.MAC file on the

HDS-300 diskette in drive B: of

the IBM PC/AT to a DOS file on

the current drive/directory.

HDSTYPE      Display HDS-300 file contents

------------------------------------------

Description:

The HDSTYPE command allows the contents of files on a HDS-300

diskette to be displayed.  It follows a command format similar

to the MS-DOS "TYPE" command.  With this command, HDS-300 text

files (.MAC, .SCR, .LOG) may be examined on the HDS-300 diskette

without first copying them over to MS-DOS.

Syntax:

HDSTYPE  <drive>[user]:[<filespec>]

where:

<drive>      selects the IBM PC/AT drive in which the HDS-300

diskette has been inserted.

<user>       is the optional CP/M-68K user number for <filespec>.

Legal values are 0-15, with 0 as the default.

<filespec>   is an HDS-300 filename to be read.

Examples:

HDSTYPE  A:USERINIT.MAC      Displays the USERINIT.MAC file on

the HDS-300 diskette in drive A:

of the IBM PC/AT on the console.

HDSTYPE  A15:TEST39.SCR      Displays the TEST39.SCR file under

user 15 on the HDS-300 diskette in

drive A: of the IBM PC/AT on the

console.

HDSWRITE     Copy DOS file to HDS-300 Diskette

----------------------------------------------

Description:

The HDSWRITE command allows DOS files to be copied to a

HDS-300 diskette.  It follows a command format similar to the

MS-DOS "COPY" command.

Syntax:

HDSWRITE  <DOS_path>  <drive>[user]:[<filespec>]

where:

<DOS_path>   is the MS-DOS source path to be read.

<drive>      selects the IBM PC/AT drive in which the HDS-300

diskette has been inserted.

<user>       is the optional CP/M-68K user number for <filespec>.

Legal values are 0-15, with 0 as the default.

<filespec>   is an HDS-300 filename to be written.

Examples:

HDSWRITE  C:USERINIT.MAC  A:   Copies the USERINIT.MAC file on

DOS drive C: to the HDS-300

diskette in drive A: using the

same filename.

HDSWRITE  TMP.MAC  A5:TST.MAC  Copies the TMP.MAC file on the

default DOS drive to the HDS-300

diskette in drive A: under user

number 5 using the filename

TST.MAC.

4. Restrictions

a. It is not physically possible to format an HDS-300 diskette in

an IBM PC/AT drive.  Therefore, all formatting of HDS-300 disk-

ettes MUST be done on the HDS-300 station using the supplied

OFFLINE UTILITIES diskette for the matching SYSTEMS disk.  The

boot track was changed for later releases, so to ensure a boot-

able diskette, always use the matching UTILITIES diskette.

b. When using a DOS based editor to edit HDS-300 macro files, the

following cautions should be noted:

1. A proper HDS-300 macro file is terminated with a CONTROL-Z

(^Z) end-of-file character.  It is preferred to always end

a macro file with a string of "******"s starting in column

one.  This prevents possible end-of-file problems.

2. A macro file can only have up thru 78 characters on any

single line.  Some older versions only allow up to 77 char-

acters per line.  Lines longer than the allowed limit will

cause macro execution to halt.  Expansion of local symbols

can also cause lines to become too long, so be careful.

3. The HDS-300 software only accesses files under user number

0.  Therefore, any files written to other user numbers will

not be accessible by the HDS-300 software.  User number

capability is provided for general CP/M-68K support.

**EOF**