DISKTIME.PAS

------------

Program DISKTIME is used to measure the average access time on the

IBM-PC/XT and the IBM-PC/AT fixed disk.  It is written in Turbo Pascal 3.0 as

distributed by Borland International, 4585 Scotts Valley Drive, Scotts Valley,

California, USA, 95066.  This program will not destroy the data residing on

your hard disk(s), and communicates solely through BIOS service interrupt 13h.

Comparison times are from Table 5, Page 171, No 11, Vol 10 (Nov 85) of BYTE

and are increased by 8.33 milliseconds to compensate for disk latency.  If you

have problems on a compatible, then it is not as compatible as you thought...

Permission is freely given by the Author of this program to distribute

copies of the UNMODIFIED program as you see fit.  Please note that you will

have to run ARC(hive) (included) to extract the sources from the library.  Here

is how you should proceed to generate a copy from sources

N>arc e disktime                    - Extract all sources

N>turbo

TURBO PASCAL SYSTEM       VERSION N.NN

COPYRIGHT (C) 1983,1984,1985 BY BORLAND INC

INCLUDE ERROR MESSAGES (Y/N)? y      - Want diagnostics

LOGGED DRIVE: N

WORK FILE:

MAIN FILE:

EDIT      COMPILE  RUN    SAVE

DIR       QUIT  COMPILER  OPTIONS

>O                                   - Option change

COMPILE -> MEMORY

COM-FILE

CHN-FILE

COMMAND LINE PARAMETER

c                                    - Compile to disk

COMMAND LINE PARAMETER

q                                    - Main menu

LOGGED DRIVE: N

WORK FILE:

MAIN FILE:

EDIT      COMPILE  RUN    SAVE

DIR       QUIT  COMPILER  OPTIONS

>c                                   - Compile program

WORK FILE: disktime                  - Enter name

COMPILING --> N:\DISKTIME.PAS

LOGGED DRIVE: N

WORK FILE:

MAIN FILE:

EDIT      COMPILE  RUN    SAVE

DIR       QUIT  COMPILER  OPTIONS

q                                    - Exit Turbo Pascal

N>

The reason I wrote DISKTIME was to demonstrate how significant the access

time is on a small computer system, and to encourage the development of

a stand-alone utility which would compensate for this.  Please note that

MS-DOS allocates file space in "Clusters" of 2k blocks each, which are not

necessarily consecutive.  After you have been using your hard disk for

a while, sequential file clusters will probably be scattered at random

throughout your hard disk, and the time to load a file or program will

therefore be approximately equal to (ACCESS TIME) * (NUMBER of CLUSTERS)

For the 100 kilobyte Microsoft assembler, this works out to a potential

of (73.3 milliseconds)*(50 clusters) or approximately four (4) seconds

on the IBM-PC/AT !!!  Hence the urgency in my appeal for de-fragmenters.

Ideally such a disk optimization facility would be well-behaved, in that

o It would be stand-alone, not requiring operator intervention

o It would require no additional peripherals such as floppies or tape

o It would not require scratch space on the disk being optimized

o It would leave the disk in a usable state after abnormal termination

Finally, I should like to thank the authors of the ARC(hive) librarian

System Enhancement Associates

21 New Street

Wayne, N.J.

USA     07470

and to note that it is easier to use, friendlier, and produces smaller files

than the LU utility which the Turbo users group currently supports...

Sincerely,

Y.N. Miles

c/o TRIUMF

4004 Wesbrook Mall

Vancouver, B.C.

Canada, V5Y 1Z7

Ph (604) 222-1047