Programming the Sound Blaster DSP in DMA Mode

(Note: All values are hexadecimal. This information was obtained

from the Book "Sound Blaster The Official Book" ISBN 0-07-881907-05)

1.    Set up the ISR

2.    Program the sampling rate to the DSP

a. Turn on speaker (write D1 to DSP command register 2xC)

b. Send the Set-Time Constant command (write 40 to 2xC)

c. Send the time constant (write byte to DSP command register 2xC)

3.    Program the DMA Controller for the first data block

a. Select DMA channel 1 by writing 5 to DMA mask register 0A

b. Reset DMA transfer flip/flop (write 00 to DMA clear byte pointer

register 0C)

c. Select DMA mode for DAC output (write 45 to DMA mode register 0B)

d. Send data memory address to DMA controller (write lo byte to 02

and hi byte to 02)

e. Send physical memory page number to DMA controller (write to

page address register 83)

f. Send DMA data-byte (data length -1) to ch1 transfer count

register  03 lo then hi

g. Enable DMA channel 1 (write 01 to mask register 0A)

4.    Program the DSP for the first data block

a. Send DMA mode 8-bit DAC command (write 14 to DSP command

register 2xC)

b. Send the length-1 of data to be transfered (lo, then hi to 2xC)

5.    Service the DSP Interrupt

a. Save machine status (registers)

b. Acknowledge DSP interrupt by reading DSP data available status

register 2xE

c. If no more blocks to transfere, goto step 5f

d. Program the DMA controller for next data block (step 3)

e. Program DSP for next data block (step 4)

f. Output End-Of-Interrupt to PIC (execute EOI OUT 20,20)

g. Restore registers.

h. execute IRET

6.    Restore original ISR