File: BLUE BOXING INFORMATION

Read 27 times

[1-34, Last=32, Quit=Q] Read File #>32

Section: Phreak II

[16],[?][Q]=Quit, [G-Sec Cmd]: 9

Now in: Phreak II section

{}{}{}{}{}{}{}{}{}{}{}{}{}{}{}{}{}{}{}{

Section: Phreak II

[16],[?][Q]=Quit, [G-Sec Cmd]: S

)=-> Read Phreak II Section <-=(

Phreak II section

status: 34 files. Please wait...

Start listing titles with <1> #32

Increment Titles by <10>: )=->

[32]  BLUE BOXING INFORMATION

[33]  YET MORE PHUN WITH UNIX

[34]  PBX'S & EXTENDERS

<T> [R] [Q] [?] ->32  R

[1-34, Last=32, Quit=Q] Read File #32

File: BLUE BOXING INFORMATION

Read 28 times

BLUE BOXING

[===========]

THIS IS THE TONE MATRIX FOR A BOX WHICH GENERATES TONES THAT

OPERATORS USE TO DIAL..ROTARY WORKS AS WELL, ON OPERATOR LINES,

BUT THIS IS TECHNOLOGICAL(!). NOW I AGREE WITH THE OPINION O F A

WELL KNOWN PHREAK THAT 'BOXING' IS/WILL BE FOR THE MOST PART

DEAD, BUT THIS IS TRADITION... FIRST,YOU DIAL DIR.ASST, OR AN

OPER. ETC , THEN YOU BLAST THE LINE WITH A 2600HZ TONE. THIS

GIVES YOU THE LINE, THIS IS ALSO HOW MA BELL TRACKS DOWN BLUE

BOXERS... THERE  ARE 2600HZ DETECTORS SY STEMS, AND EVEN ON OLD

#4 CROSSBARS... ONCE ON A OPER.TRUNK LINE, YOU USE YOUR BLUE

BOX/ROTARY TDO DIAL...

SO,  IF YOU USE 2600HZ, WHICH IS NECESSARY, UNLESS YOU ARE

*VERY* CAREFUL, YOU WILL BE SNAGGED.

FINALLY, THIS IS WHAT YOU READ! SO LONG

AND HARD FOR:

700   :   1   :   2   :   4 :   7   :  11   :

900   :   +   :   3   :   5 :   8   :  12   :

1100  :   +   :   +   :   6 :   9   :  KP   :

1300  :   +   :   +   :   + :  10   :  KP2  :

1500  :   +   :   +   :   + :   +   :  ST   :

:  900  : 1100  : 1300 : 1500 : 1700  :

USE KP TO START A CALL, AND ST TO STOP, WITH THE BELOVED 2600HZ

TONE TO DIS CONNECT. I ALSO HEAR THAT 2600HZ RESETS SPRINT NODES

AND GIVES YOU THEIR INITIAL TONE..

NOW, IF YOU'RE WONDERING ABOUT WHAT TO CALL FROM AN

OPERATOR TRUNK, HERE ARE SOME GOODIES TO HELP YOU OUT:

XXX+101 - TOLL  SWITCHING

XXX+121 - LOCAL OPERATOR

XXX+131 - INFORMATION

XXX+141 - RATE & ROUTE

XXX+181 - COIN REFUND OPERATOR

XXX+11501 - MOBILE OPERATOR

XXX+11521 - MOBILE OPERATOR

XXX+11511 - CONFERENCE OPERATOR

---CONCERNING ABOVE

FIRST THE TONES.   WHILE ALL THE INFORMATION IS CORRECT,

THE TIMING SPECS WERE NOT INCLUDED .   THE TONE PAIRS ARE TO

REMAIN ON FOR 1/10 SEC. WITH 1/10 SEC. OF SLIENCE BETWEEN

DIGITS.  THE 'KP' TONES  SHOULD BE SENT FOR 2/10 SEC.   A WAY TO

DEFEAT THE  2600HZ TRAPS  IS TO SEND ALONG WITH THE 2600HZ

SOME PINK  NOISE(MOST OF THE  ENERGY  IN THIS SIGNAL SHOULD BE

ABOVE 3000HZ,  THIS  SIGNAL WON'T MAKE IT OVER THE TOLL NETWORK,

BUT SHOULD CARRY AS FAR AS YOUR  LOCAL  TOLL  CENTER SO THAT THE

TRAPS  WON'T  FIND  'PURE' 2600HZ  ON THE TRUNK. THIS IS NOT A

PERFECTLY SAFE WAY TO BOX, BUT IT SHOULD SLOW DOWN THE DISCOVERY.

AS  TO USE,  THE FIRST THING YOU NEED TO UNDERSTAND IS THAT

THERE  ARE  TWO(2)  TYPES OF TOLL COMPLETING  TRUNKS: INWARD AND

OUTWARD.  THE NAMES ARE REFERENCE TO THE OFFICE THAT IS SWITCHING

THE  CALL(THE TOLL CENTER THAT SERVES THE WATS LINE  YOU  CALLED)

AND EACH TYPE OF TRUNK HAS A DIFFERENT CLASS OF SERVICE.  FROM

AN INWARD TOLL COMPLETING TRUNK, YOU CAN REACH THE DIFFERENT

SERVICE OPERATORS,  THE TOLL TEST BOARD, AND THE INWARD OPERATOR.

SOME OFFICES ALSO ALLOW REMOTE TESTING AND IT IS IN THESE OFFICES

THAT YOU  CAN ACCESS THE OUTWARD TOLL COMPLETING TRUNKS.  THE OUT

WARD TRUNKS  ALLOW  YOU  TO MAKE VERIFICATION(EMERGENCY)  CALLS,

DO SERVICE MONITORING(TAPPING), STACK TRUNKS(BUSY OUT ALL TRUNKS

BETWEEN LA AND NYC),  ENABLE AND DISABLE TSPS POSITIONS,  AN D IN

SOME CASES(ON SOME 4A'S) ISSUE TEMPORARY REROUTING INSTRUCTIONS(

SEND  ALL CALLS FROM LA TO NYC VIA MIAMI,  BOSTON,  OR ANY OTHER

CLASS 5 OFFICE OR OFFICES). BOTH TYPE OF TRUNKS ALLOW YOU TO

PLACE A 'STANDARD' CALL WITH A BOX.

IN  SOME  OFFICES,  MOSTLY THE SMALL ONES WITH A  TOLL TEST

BOARD THAT IS UNATTENDED AT NIGHT AND ON WEEKENDS, YOU CAN GET

AN OUTWARD TOLL COMPLETING TRUNK AS WELL AS PERFORMING OTHER TEST

AND ROUTING FUNCTIONS.  YOU DO THIS BY USING THREE DIGIT  CODES

THAT  ARE INVALID EXCHANGES(NOT OF THE PATTERN NNX[SEE NOTE 1]).

DURING THE SIXITES  THE  CODES USED WERE FAIRLY  STANDARD AND

CONSISTENT, HOWEVER WHEN THE BOXES BECAME POPULAR AND THE

PHREAKS STARTED DOING THINGS LIKE ROUTING ALL CALLS FROM DALLAS

TO FT. WORTH VIA WASHINGTON D.C. MOTHER STARTED CHANGING THE

TEST CODES ON  A  RANDOM(AS FAR AS I KNOW ) BASIS.  WHAT I WOULD

SUGGEST IS THAT EVERYBODY INTERESTED IN DOING THIS SORT OF THING

PICK OUT A NICE  QUIET  LITTLE OFFICE SOMEWHERE AND WORK ON

DISCOVERING THE CODES ACCEPTABLE TO THAT OFFICE.

EACH  NUMBERING PLAN AREA (NPA,  ALSO KNOWN AS AREA CODE)

HAS AN OFFICE DESIIGNATED AS ITS MASTER OFFICE.  THIS OFFICE CONT

ROLS ALL OF THE OTHER TOLL OFFICES IN THE AREA AS WELL AS SERVING

AS A CONCENTRATION POINT FOR MOST OUT OF AREA CALLS. TO ACCESS

THE SERVICES OF A NON-MASTER OFFICE YOU NEED IT'S 'CITY  CODE',

THIS IS  A  THREE(3) DIGIT CODE THAT IS OF THE FORM 0XX,  AND IS

SENT AFTER THE AREA CODE[SEE NOTE 2 ].  AS AN EXAMPLE,  THE 'CITY

CODE' FOR CANTON,  OHIO IS 042;  TH US TO REACH THE INWARD

OPERATOR IN CANTON, YOU WOULD SEND 'KP-216 -042-121-ST' WHERE AS

IF YOU WANTED THE INWARD OPERATOR IN CLEVELA ND, YOU WOULD SEND

'KP-216-121- ST'. THE  REASON THIS IS NECESSARY IS THAT THE

OPERATOR IN  CLEVELAND CAN'T VERIFY A NUMBER IN CANTON, SO IF

YOU WANT TO VERIFY SOMEONE IN CANTON YOU NEED THE CITY CO DE.

ALSO, MOST AREA MASTER OFFICES HAVE DEDICATED DATA TRUNKS TO THE

NETWORK CONTROL CENTER AND THUS DON'T  ACCEPT TEST AND REROUTING

COMMANDS OVER THE SWITCHED NETWORK.

IN CONCLUSION,  THE SWITCHING NETWORK WILL DO A LOT MORE

FOR YOU THEN CONNECT YOU TO PEOPLE AND THE SMALL OFFICES THAT

REQUIRE A 'CITY CODE' ARE THE TYPE OF OFFICE TO TRY TO BREAK.

NOTE 1:   THE  NORMAL  FORMAT  FOR TELEPHONE NUMBERS  IS

AS FOLLOWS: NYN/NNX-XXXX.  WHERE N=ANY DIGIT EXCE PT 1 AND 0;

Y=0 OR 1 , AND X=ANY DIGIT.  YES I KNOW THAT IN SOME AREA

CODES THE NNX FORMAT HAS CHANGED TO NXX.  THIS IS A NEW

OCCURRENCE AND ONLY OCCUR WHERE THERE HAS BE AN OUTRAGEOUS

POPULATION INCREASE IN THE LAST FEW YEARS AND ALL  OF THE FUNNY

EXCHANGES ARE CONNECTED DIRECTLY TO  MASTER OFFICES AND THUS

DON'T CONFLICT WITH THE 'CITY CODE' FORMAT.

NOTE 2:   YOU CAN OBTAIN THE 'CITY CODE' FOR A NUMBER BY

CALLING RATE AND ROUTE AND ASKING FOR THE 'NUMBERS ROUTE' TO

NYN/NNX(I.E. 914/725). OR IF YOU LEAVE ME A MESSAGE WITH THE

AREA CODE AND FIRST THREE OF A NUMBER, I WILL GET YOU THE

'CITY CODE'.

-END

[1-34, Last=32, Quit=Q] Read File #

###########